# Manufacturing Plant – Network Topology

IPSec P2P VPN (Plant↔DC)  |  Cisco AnyConnect → DC ASA 5545-X DIRECTLY  |  MS AD/LDAP Auth  |  Cisco ASA 5515-X / 5545-X  |  Nexus 9000  |  VMware ESXi 7.x  |  IBM Power8  |  Dell R630

⚠  TWO SEPARATE VPN PATHS:
  PATH 1 – SITE-TO-SITE:   Plant users/systems → Plant ASA 5515-X → IPSec P2P Tunnel → DC ASA 5545-X
  PATH 2 – REMOTE ACCESS:  Remote users (AnyConnect) → Internet → DC ASA 5545-X  DIRECTLY  (Manufacturing Plant is NOT involved)

REMOTE USERS (AnyConnect)                        MANUFACTURING PLANT SITE
  ================================                 ==========================================
  +------------------------------+                 +--------------------------------------+
  |  Remote Users / Roaming      |                 | Plant Floor / Devices / IoT          |
  |  (Cisco AnyConnect Client)   |                 +--------------------------------------+
  |  Laptop / PC                 |                 | Office LAN / Workstations            |
  +-------------+----------------+                 +--------------------------------------+
                |                                  | OT / SCADA Network                   |
                | SSL/TLS TCP 443                  +------------------+-------------------+
                | via Internet                                        |
                | (goes directly to DC)            +-----------------+--------------------+
                |                                  | Cisco Nexus 9000 - Plant Core Switch |
                | ** Plant ASA 5515-X is           +-----------------+--------------------+
                |    NOT in this path **                              |
                |                                  +-----------------+--------------------+
                |                                  | Cisco ASA 5515-X                     |
                |                                  | Edge Firewall / IPSec P2P VPN GW     |
                |                                  | (Site-to-Site only, not AnyConnect)  |
                |                                  +-----------------+--------------------+
                |                                                    |
                |                                                   IPSec P2P Tunnel
                |                                                    |
                +------------------------------------+               |
                                     |              |               |
                                     v              v               v
  ================================================================================
                              DATACENTER SITE
  ================================================================================

  +------------------------------------------------------------------+
  |                    Cisco ASA 5545-X                               |
  |  -------------------------------------------------------------- |
  |  [1] DC Perimeter Firewall                                       |
  |  [2] IPSec P2P VPN Termination  <-- from Plant ASA 5515-X       |
  |  [3] AnyConnect SSL VPN Server  <-- Remote Users (TCP 443)      |
  |      Authentication: MS Active Directory via LDAP / LDAPS       |
  +-----------------------------------+------------------------------+
                                      |
  +-----------------------------------+------------------------------+
  |           Cisco Nexus 9000  -  DC Core Switch                    |
  +----------+-----------------------+-----------------------+--------+
             |                       |                       |
  +----------+--------+  +-----------+-----------+  +-------+-----------+
  | IBM Power8 Servers |  | Dell R630 / ESXi 7.x  |  | SAN (FC/iSCSI)   |
  | (Bare Metal)       |  |                       |  |                  |
  |                   |  | VM: vCenter Server    |  | VMFS Datastores  |
  | SAP App Server    |  | VM: MS Exchange       |  | Nakivo Repo      |
  | SAP DB (DB2/HANA) |  | VM: MS SQL Server     |  +------------------+
  |                   |  | VM: Active Dir / DNS  |<-- ASA LDAP Auth
  | iDRAC OOB (NIC)   |  | VM: MS SharePoint     |  +------------------+
  +-------------------+  |   +RFQ/Quotation App  |  | Nakivo B&R       |
                         | VM: File/Print Server |  | VM/ESXi Backup   |
                         | VM: Other MS Apps     |  | Replication      |
                         | iDRAC OOB per Host    |  +------------------+
                         +-----------------------+

  +------------------------------------------------------------------------+
  | Out-of-Band Mgmt Network (Dedicated VLAN)                               |
  | [ IBM iDRAC ]  --  [ Dell R630 iDRAC x N ]  --  [ Jump Host/Bastion ] |
  +------------------------------------------------------------------------+

# Cisco AnyConnect Remote Access VPN – Authentication Flow

AnyConnect remote users connect DIRECTLY to the Datacenter Cisco ASA 5545-X over the internet. The Manufacturing Plant ASA 5515-X is NOT involved in the AnyConnect path.

Remote user launches Cisco AnyConnect VPN Client on their laptop/PC.

Client connects DIRECTLY to the Cisco ASA 5545-X public IP on TCP 443 (SSL/TLS) over the internet. The Manufacturing Plant ASA 5515-X is NOT in this path.

The ASA 5545-X presents its SSL certificate and challenges the user for credentials.

The ASA 5545-X forwards credentials to VM: Active Directory / DNS via LDAP (port 389) or LDAPS (port 636).

Active Directory validates the credentials and returns success or failure to the ASA.

On success, the ASA 5545-X establishes the AnyConnect tunnel and assigns the user an IP from the remote-access pool.

User traffic is routed through the tunnel into the datacenter per ASA ACL and group policies.

Split-tunneling policy determines which traffic goes through the VPN vs. direct internet.

Security Recommendation: Enforce MFA (Cisco DUO or Microsoft Authenticator) on AnyConnect logins. Use a read-only AD service account for LDAP queries from the ASA.

# Component Summary

| Layer | Component | Details |
| --- | --- | --- |
| Remote Access | Cisco AnyConnect VPN Client | Connects DIRECTLY to DC ASA 5545-X on TCP 443 (SSL) via internet. Plant ASA 5515-X is NOT involved. |
| Remote Auth | MS Active Directory / LDAP | ASA 5545-X queries AD via LDAP/LDAPS to authenticate AnyConnect users. Group policy per AD group. |
| Plant Core Switch | Cisco Nexus 9000 | Layer 3 core. VLANs for OT, Office, Plant Floor. |
| Plant Edge Firewall | Cisco ASA 5515-X | Stateful firewall. IPSec P2P VPN gateway to datacenter only. NOT involved in AnyConnect. |
| WAN / Site-to-Site VPN | IPSec Point-to-Point Tunnel | Encrypted tunnel: ASA 5515-X (Plant) <-> ASA 5545-X (DC) only. |
| DC Edge / VPN Server | Cisco ASA 5545-X | (1) Terminates IPSec P2P from plant. (2) AnyConnect SSL VPN server for remote users. (3) LDAP auth to AD. (4) DC perimeter firewall. |
| DC Core Switch | Cisco Nexus 9000 | DC fabric; connects all server tiers and SAN. |
| SAP Tier | IBM Power8 (Bare Metal) | SAP Application Server + SAP Database (DB2 / HANA). |
| Virtualization Mgmt | VMware vCenter Server (VM) | Centralized ESXi host management, HA/DRS. |
| MS Applications | Dell R630 + VMware ESXi 7.x | Exchange, SQL Server, Active Directory/DNS (LDAP), SharePoint, File/Print. |
| Business App | Custom RFQ/Quotation System | Built on MS SharePoint. |
| Storage | SAN (FC / iSCSI) | VMware VMFS datastores + Nakivo backup repository. |
| Backup | Nakivo Backup & Replication | VM backups, ESXi host backup, replication. |
| OOB Management | iDRAC per Server (Dedicated NIC) | Isolated OOB VLAN. IBM Power8 & Dell R630. Jump host access only. |